Ystradowen Community and Sports Association

Fire Risk Assessment

Please don't hesitate to read this document, but do not remove it from the hall.

If you would like your own copy it is available to download from our website or contact 07494 555125

Ystradowen Community and Sports Association Fire Risk Assessment

Premises Address:

Ystradowen Village Hall Motte Field Ystradowen

Cowbridge CF71 7SY

Responsible Person                       David Thursfield

Risk Assessment Carried Out By   David Thursfield Date	15th October 2025

Review Date:	Annually unless there are any other significant changes or incidents

Policy:

It is the policy of the YCSA Committee to protect all persons using the premises from the risks associated from any potential fire.

Premises

The building is single floor Main Hall and Annex with walls constructed from brick and block, with a flat roof constructed of steel truss girders (in Main Hall) and timber “Easijoists (in Annex), overlaid by plywood, 150mm of insulation, and waterproofed by a single ply PVC membrane (tradename Fatra).

There are no stairways.

Premises consist of 4 main meeting rooms of various sizes, kitchens, w/c facilities and storage rooms.

Occupancy

08.00 to 24.00

No one lives in the premises Maximum Permitted numbers

Room	Numbers	Area (m2)

| Main Hall | 140 | 113 |
| --- | --- | --- |
| Conference Room | 28 | 28 |
| Annex 1 | 55 | 55 |
| Annex 2 | 22 | 22 |

Children's Parties (<11's). Maximum number of children in the hall is 30 with 20 Adults helping / supervising

NB the occupancy number depends on the type of use.

Risk of fire is considered to be low and it is expected that complete evacuation of the premises could be achieved within 2 minutes of an alarm.

The hall is hired by a wide range of various groups. These vary from a retired group of village pensioners, quilting groups, lace groups, ballroom dancers. There is a younger group of ballet dancers, Pilates and Brownies. There is a Coworker networking group in the Annexe. The YCSA also puts on events such as quiz nights and skittles evenings and there is a Cinema Club.

Fire Warning Systems:

Automatic detectors in Annex and in Main Hall. and heat alarm in kitchen. All alarms (Main Hall and Annex) are interlinked.

Emergency Lighting.	3 hours BS 5266

Sprinkler System	No

Fire Hazards

Sources of Ignition: The hall is protected by 2 up to date consumer units with a 30mA Electrical RCD and appropriate MCB's. The main hall consumer unit is located in a high-level cupboard in the meeting room, and the Annex consumer unit is located in the storage cupboard off the hall corridor.

The electrical system is inspected and tested by an approved electrician at the prescribed intervals. This includes items such as the tea urn, kettle, fridge and boilers.

Sources of Power:	The entire building is powered by a combination of a mains 3 phase electricity supply, and s 30 KVA solar panel array mounted on the Hall roof. The incoming mains and solar supply are in the meeting room within the main hall. There are also 28KWH of solar storage batteries and a 30KVA inverter located within a ventilated locked cupboard within the meeting room.

There is a terminated gas supply to the kitchen but this has been sealed off and capped under the kitchen sink unit.

The building is heated by 2 externally located electrically powered heat pumps (located behind the main store), with buffer tanks and controls in a locked cupboard within the main store at the south end of the main hall.

There are no thermostats within the building, the heating temperature is controlled via mobile phone App and thermostatic receivers mounted on the radiators.

Automated Hall Entry System

Both the Main Hall and the Annex entrance doors and alarms are controlled by a “Paxton 10” automated entry system.  Entry can be gained using an electronic key fob or by keying in a unique code which is provided to every hall user. The key pads are located by each of the entrance doors.

Other general items within a village hall as used by the various groups using the hall. For example, chairs, books, judo mats. These are all kept in a clean and ordered manner.

People at Risk:	All hall users are potentially at risk.

All users should be briefed on the action to take in the event of a fire.

There are 2 means of exit from the main hall and 2 exits from the Annex. This is in compliance with the Building Regulations.

Any fire would be considered as a small to medium growth and would be noticed soon after outbreak due to the open plan nature and low flammability of the premises.

All doors can be opened easily and escape routes lead to a place of safety. All escape routes are covered by emergency lighting.

Fire Safety Signs.	In place and indicate the fire exits. Fire Warning System.	In place.

Emergency Lighting.	There is adequate emergency lighting in the premises which

is checked annually.

Fire Fighting Equipment

There are a sufficient number of extinguishers and fire blankets sited throughout the premises which are adequate for the risks within the premises. These are serviced annually.

Specific Fire Arrangements

For groups having a theatre style layout, for example when watching a play or the Cinema Club:

Permitted Numbers.

This is 115. Numbers will be controlled by the group organiser.

Room layout.

There should be a clear space of at least 305mm between the back of one

seat to the front of the seat behind it. Gangways should be at least 1.05m wide. There should be no projections which diminish these widths.

No seat should be more than seven seats away from a gangway. If temporary seating is provided, these should be secured in lengths of not fewer than four seats (and not more than 12).	Cable ties can be used to secure the seating.

General:

An evacuation announcement as part of the introductions indicating the exits to use (2 of them) in the event of an alarm.

Location of Mechanical and Electrical Equipment: Electrical

The consumer unit for the main hall is located in the cupboard in the corner of the Conference Room (behind the main doors).

The consumer unit for the annex is located in the store cupboard situated off the hall corridor just beyond the kitchen area before the large room.

Gas

As stated above, the gas supply has been sealed and capped off in the cupboard below the sink unit in the main hall kitchen

Actions Required Following This Risk Assessment

Actions to take.

Advise all users of the fire evacuation procedure, exits and assembly point. Advise on use of fire fighting equipment.

Regular groups should have a fire drill.

A logbook to be maintained detailing electrical, and fire fighting equipment maintenance. Also to record emergency lighting checks and any incidents

YCSA Village Hall Emergency Action Plan

It is important that you as the designated hall hirer know what to do in the event of a fire. You will be in charge until the fire brigade arrive.

You should make sure all your group know what to do in the event of a fire, for example, pointing out the fire exits.

For groups that meet monthly, at the beginning of your activity make a general announcement reminding your group where the exits are and what to do if the alarms sound.

For groups meeting on a weekly basis, make a general announcement reminding your group where the exits are and what to do if the alarms sound. Please also consider carrying out a practice fire drill at 6 monthly intervals.

Do not bring anything into the hall that could be a source of ignition or be flammable. For example balloons filled with flammable gasses, battery operated igniters, flammable paints and sprays.

If you bring your own electrical equipment please tell us so we can arrange to get it electrically safety checked for you at a nominal cost.

In the unlikely event of a fire your first duty is to evacuate your group out of the premises. Leave any equipment you may have brought with you. If it is electrical, if possible turn it off.

Only tackle a small fire if you feel competent to do so. If you've never let off a fire extinguisher or fought a fire, this is not the time to try it for the first time.

Gather your group at the assembly point and account for everyone.

When the fire brigade arrive they will ask you if there is anyone trapped in the premises.

YCSA Village Hall Emergency Action Plan

Action to take in the event of a fire.

Assembly Point

5- a-side pitch or grassed area outside of the premises, well away from the building

Action on Discovery of a fire or the alarm sound

- Alert all users in both the Main Hall and the Annex.

- Leave the building by the nearest exit.

- Do not re enter the building until you are told it is safe to do so.

- Report to the assembly point.

- Call the emergency services.

- Only tackle small fires if you are competent to do so. The hall is insured and can be replaced - you cannot.

- Do not put yourself at risk.

Document History

| Prepared: | Chris Kipling | 10th June 2019 |
| --- | --- | --- |
| Issued: Reviewed |  | 15th July 2019 18th August 2020 |

- Permitted numbers reviewed

- Additional column added "Maximum Numbers During CV19" Reviewed	9th October 2021

- Permitted numbers reviewed in light of current Corona virus information including children's parties.

- Need to re-emphasize to users the importance of briefing their groups when they meet about what to do in the event of a fire and for practice evacuations.

- Fit smoke alarms in the main hall.

- Noted locations of M&E equipment.

Re-Issued	22nd March 2022 Re-Issued	 7th August 2022

Now reflects that there are smoke alarms, CO detector and heat sensors fitted in the Main Hall.

Re-Issued	13th March 2023

References to CV19 now removed

Fire alarm certificate renewed on 24/4/24 Emergency Lighting certificate renewed 27/4/24

Reviewed        Termination of gas supply noted.

Introduction of Heat Pumps, Solar panels and 3 phase electricity noted.

Automated entry system noted.

Reviewed by David Thursfield 15th October 2025.